1. INTRODUCTION

1.1 General

Soil is the aggregation of organic and inorganic materials on the surface of the earth that provides the medium for plant growth. Soil has been developed slowly over time composed of many different materials. Soil is a vital part of the natural environment and it influences the distribution of plant species and provides a habitat for a wide range of organisms (McAluffie.,1994). It controls the flow of water and chemical substances between the atmosphere and the earth, and acts as both a source and storage for gases (like oxygen and carbon dioxide) in the atmosphere. Soils not only reflect natural processes but also record human activities both at present and in the past. They are therefore part of our cultural heritage.

A given volume of soil consists of four parts: mineral matter, organic matter, water, and air .The mineral and organic matter of a soil store nutrients required by crops. Soil acts as a filter for underground water, filtering out pollutants. It provides all the nutrients required for successful plant growth. Soil consists of 45% minerals, 25% water, 25% air & 5% organic matter (Cameron.,2005).

Figure 1.1.The Composition of Soil

The formation of soils can be seen as a combination of the products of weathering, of structural development of the soil, of differentiation of that structure into horizons or layers, and lastly of its movement or translocation. Soil has 6 layers called horizons O, A, E, B, C and R. Horizon O is the topsoil and R is bedrock().

1.2PHYSICAL PROPERTIES OF SOIL:

Soils are porous and open bodies, yet they retain water. They contain mineral particles of many shapes and sizes and organic material which is colloidal in character. The solid particles lie in contact one with the other, but they are seldom packed as closely together as possible.

The following are some of the physical properties of soil:

### 1.2.1. Texture

The size distribution of primary mineral particles, called soil texture, has a strong influence on the properties of a soil. The amount of sand, clay and silt is what gives different soil types their various textures. Most soils are a mix of all three (Pierre-Yves Hicher.,1996).

Particles smaller than 2 mm in diameter are divided into three broad categories based on size: particles of 2 to 0.05 mm diameter are called sand; those of 0.05 to 0.002 mm diameter are silt; and the <0.002 mm particles are clay. The texture of soils is usually expressed in terms of the percentages of sand, silt, and clay.

### 1.2.2. Structure

The nature of the arrangement of primary particles into naturally formed secondary particles, called aggregates, is soil structure. Structure may be further described in terms of the size and stability of aggregates. Structural class is based on aggregate size, while structural grade is based on aggregate strength.

### 1.2.3. Consistency

Consistency is a description of a soil's physical condition at various moisture contents as evidenced by the behavior of the soil to mechanical stress or manipulation. Descriptive terms such as hard, loose, friable, firm, plastic, and sticky are used to denote the consistence of soil(Pierre-Yves Hicher.,1996). The consistence of a soil is determined to a large extent by the texture of the soil, but is related also to other properties such as content of organic matter and type of clay minerals.

### 1.2.4. Colour

Generally, moist soils are darker than dry ones and the organic component also makes soils darker. Thus, the surface soil tends to be darker than subsoil. Red, yellow and gray hues of subsoil reflect the oxidation and hydration states or iron oxides present in the subsoil.

### 1.2.5. Particle Density

Particle density is the weight of a given soil particle per unit volume. In other words, it is the weight of a soil particle or separate, divided by the volume of that soil particle or separate. In most mineral soils, the average particle density is between 2.6 and 2.7.

### 1.2.6. Bulk Density

Bulk density of a soil is defined as the weight per unit volume of soil. A unit volume of soil includes both the solids and the pore space. Bulk density is important because it reflects the porosity of a soil. Loose, porous soils have lesser bulk densities than tight, compacted soil(Pierre-Yves Hicher.,1996).The bulk density of a soil increases with compaction. Bulk density indicates how easily a soil will till, how easily water will infiltrate, how it will hold water, and its suitability for growing plants.

### 1.2.7. Soil Porosity

The space between soil particles is the pore space. This pore space contains varying amounts of water and air. Soil porosity depends on soil texture and structure. Soils with lesser bulk densities have greater porosities. Good porosity is essential to adequate soil aeration, water drainage and root penetration.

## 1.2.8. Soil permeability:

Soil permeability is the ability of the soil to transmit water and air. As the soil layers or horizons vary in their characteristics, the permeability also differs from one layer to another(Pierre-Yves Hicher.,1996).Pore size, texture, structure and the presence of impervious layers such as clay pan determines the permeability of a soil. Permeability is measured in terms of permeability rate or coefficient of permeability (cm per hour, cm per day and cm per sec.).

1.3. SOIL CHEMICAL PROPERTIES:

Soils are chemically different from the rocks and minerals from which they are formed in that soils contain less of the water soluble weathering products, calcium, magnesium, sodium, and potassium, and more of the relatively insoluble elements such as iron and aluminum. Old, highly weathered soils normally have high concentrations of aluminum and iron oxides(Pierre-Yves Hicher.,1996).The organic fraction of a soil, although usually representing much less than 10% of the soil mass by weight, has a great influence on soil chemical properties.

### Some of the chemical properties of soil are as follows:

### 1.3.1.Soil pH

Soil pH implies certain characteristics that might be associated with a soil. It has a profound influence on plant growth. Soil pH affects the quantity, activity, and types of microorganisms in soils which in turn influence decomposition of crop residues, manures, sludge and other organics. It also affects other nutrient transformations and the solubility, or plant availability, of many plant essential nutrients. A soil pH of 7 is considered neutral. Soil pH values greater than 7 signify alkaline conditions, whereas those with values less than 7 indicate acidic conditions. Soil pH typically ranges from 4 to 8.5.

1.3.2. Soil Electrical Conductivity:

Soil electrical conductivity (EC) is a measure of the amount of salts in soil (salinity of soil). It is an important indicator of soil health. It affects crop yields, crop suitability, plant nutrient availability, and activity of soil microorganisms which influence key soil processes including the emission of greenhouse gases such as nitrogen oxides, methane, and carbon dioxide. Although EC does not provide a direct measurement of specific ions or salt compounds, it has been correlated to concentrations of nitrates, potassium, sodium, chloride, sulfate, and ammonia. For certain non-saline soils, determining EC can be a convenient and economical way to estimate the amount of nitrogen (N) available for plant growth.

### 1.3.3. Cation-Exchange Capacity:

Soil consists of positively charged ions called cations(Ca2+, Mg2+, K+,NH4+, Zn2+, Cu2+, and Mn2+). The cations in the soil solution are in dynamic equilibrium with the cations adsorbed on the surface of clay and organic matter. CEC is a measure of the quantity of cations that can be adsorbed and held by a soil. (Pierre-Yves Hicher.,1996).CEC is dependent upon the amount of organic matter and clay in soils and on the types of clay. In general, the higher OM and clay content, the higher the CEC.

1.4. SOIL ORGANIC MATTER

Soil organic matter (SOM) is the organic component of soil, consisting of three primary parts including small (fresh) plant residues and small living soil organisms, decomposing (active) organic matter, and stable organic matter (humus). Soil organic matter serves as a reservoir of nutrients for crops, provides soil aggregation, increases nutrient exchange, retains moisture, reduces compaction, reduces surface crusting, and increases water infiltration into soil.

Soil organic matter is composed chiefly of carbon, hydrogen, oxygen, nitrogen and smaller quantities of sulfur and other elements. The most chemically active fraction of soils consists of colloidal clays and organic matter. Soil organic matter also exists as four distinct fractions which vary widely in their size, composition and turnover times in the soil (Figure 1.2.)

- Dissolved organic matter

- Particulate organic matter

- Humus

- Resistant organic matter

Figure 1.2.Major Soil Organic Matter Components

Components vary in proportion and have many intermediate stages. Plant residues on the soil surface such as leaves, manure, or crop residue are not considered SOM and are usually removed from soil samples by sieving through a 2 mm wire mesh before analysis. Soil organic matter content can be estimated in the field and tested in a lab to provide estimates for nitrogen, phosphorus and sulphur mineralized available for crop production and adjust fertilizer recommendations. Soil organic matter impacts the rate of surface applied herbicides along with soil pH necessary to effectively control weeds. Soil organic matter impacts the potential for herbicide carryover for future crops, and amount of lime necessary to raise pH. SOM has been used to understand soil nutrient dynamics and as an ecological marker. The SOM is the major determinant and indicators of soil quality and fertility and are closely related to soil productivity in agricultural ecosystem. The reduction of SOC will lead to a decrease in soil fertility, soil nutrient supply, porosity and an increase in soil erosion. Soil organic carbon contains 58% of carbon. A factor of 1.72 can be used to convert organic carbon to soil organic matter  (Lawal et al.,2011).

1.5. TOTAL ORGANIC CARBON

Assessment of soil quality is an invaluable tool in determining the sustainability and environmental impact of agricultural ecosystems. One of the important parameters that determine the soil quality is total organic carbon.

Total organic carbon (TOC) is the organic carbon stored in soil organic matter. Organic carbon (OC) enters the soil through the decomposition of plant and animal residues, root exudates, living and dead microorganisms, and soil biota. Continuous pasture builds organic carbon quicker than other rotations.

Micro-organisms breakdown soil organic carbon as an energy source-this occurs faster when the soil is warm and moist. Cultivation can also enhance breakdown as soil aggregates are disrupted making protected organic matter available to micro-organisms to decompose and better soil aeration increases soil mobility (Nelson et al.,1996).Thus organic carbon plays a vital role in the environment through soil.By providing a food source for micro-organisms, organic carbon can help improve soil stability by micro-organisms binding soil particles together into aggregates. If the rate of organic matter addition is greater than decomposition, the organic fraction fraction in a soil will increase and vice-versa.

Total organic carbon influences many soil characteristics including colour, nutrient holding capacity, nutrient turnover and stability, which in turn influences water relations, aeration and workability. Soil organic carbon is usually expressed as a percentage of the top soil. Total organic carbon can be further defined as fractions that vary in size and decomposability. The capacity of a soil to supply nutrients is often defined by the proportion of total soil organic carbon that is active.

Total organic carbon is one of the most important constituents of the soil due to its capacity to affect plant growth as both a source of energy and a trigger for nutrient availability through mineralization.

Presently, no methods exist to measure total organic carbon in the field. Attempts have been made to develop colour charts that match colour to total organic carbon content, but the correlation is better within soil landscapes and only for limited soils. Near infrared spectroscopy has been attempted to measure carbon in the field, but it is expensive. Efforts are being made to develop a test which would directly measure carbon both economically and efficiently (Nelson et al.,1996).

2. LITERATURE REVIEW

2.1 GENERAL:

Literature review plays an important role in gaining knowledge about the project.It says how others have dealt with the similar problem and their ways to find a solution for it.

2.2- LITERATURE REVIEW 1:

2.2.1 Sorption Mechanism:

Characteristics of Soil Organic Matter and Sorption of Phenanthrene, Naphthalene 1,3,5-TCB and o-Xylene by DIYUN CHEN, WENBIAO XIE, DINGGUI LUO, AND GANG SONG at School of Environmental Science & Engineering, Guangzhou University, Guihuagang, Guangzhou, PR China

Sorbent Chelsea soil was used as it has been well characterized and employed in previous studies (Weber and Huang 1996; Xing, B et al., 1996). This soil sample was divided into four fractions, namely base-extracted soil (BE), humic acid (HA) and two sorts of humins: denser particulates (DP) and lighter particulates (LP).

HA was extracted from soil by base extraction at a soil/solution ratio of 20 g/250 mL. After the soil was extracted in 0.1 M NaOH solution for 10 h, the content of the bottle was centrifuged for 30 min at 5000 rpm, and supernatant decanted to a glass bottle. Extraction procedure was  repeated eighttimes until the supernatant became colorless.Then HA was precipitated by adjusting the supernatant solution to pH 1 with concentrated HCl.

The elemental analysis showed the Chelsea soil TOC content was 5.45%. After extraction by base, the TOC content of BE fell to 2.44%. The C and H contents and H/C atomic by base, the TOC content of BE fell to 2.44%.

The C and H contents and H/C atomic ratios were similar in different fractions, but the N and O contents and O/C atomic ratios decreased from HA to DP and to LP. Inspection of the Van Krevelen plot reveals that the DP and to LP fractions are moderately-low maturation Type 1 or 2 Kerogen, and HA approached the lignite coal rank.

DP and LP were identified by OPE in transmitted and reflected modes. The OPE (Fig. 2) showed the SOM was highly heterogeneous at the particle scale. The DP was mainly composed of huminite macerals (mainly humodetrinite), about 90% of total DP, with small amounts of inertinite (5%) and vitrinite macerals (3%). LP was composed of inertinite  (60%), huminite (10–15%), vitrinite (10%), and exinite (15%). Inertinite (including fusinite and inertodetrinite) was the dominant maceral of LP.

The sorption affinity and isotherm nonlinearity varied from sorbent-to-sorbent and sorbate-to-sorbate, which implies that the sorption behaviors of HOCs betweenwater and soil systems correlatedwith the physical–chemical characteristics of associated SOM and HOCs. The data show that: (1) Sorption isotherms can be fitted using the Freundlich model. The n < 1 and the single-point Koc value measured at low Ce isotherm n values were for a specific sorbent–sorbate system were greater than that at higher Ce values, indicatingthe sorption isotherms were nonlinear; (2) in this test the sorption 0.634–0.986. For all given sorbates, the n values were in the order HA > DP > original soil > BE > LP. The isotherm nonlinearity increased in the order HA < DP < original soil < BE < LP. This order of nonlinearity is consistent with the degree of condensed structures or glassy character expected from organic matter extraction procedures (Cuypers et al., 2002)

.

Expanded organic matter, such as HA (lower molecular weight component with higher polar functional groups), would be easily extracted compared to the insoluble condensed organic matter humin fraction (Xing and Pignatello, 1997); (3) sorption capacities (KF) varied notably from sample to sample. KF of BE were lowest and LP highest for all tested sorbents. TheKF of DP and LP were about two ordersmagnitude higher than of original soil and BE, and 1 order greater than HA, which may imply that the humin fraction in SOMs played a large role in the sorption process; and (4) the Koc calculated at a given Ce/Sw values increased in the order of HA < original soil < BE < DP < LP for all four sorbents Furthermore, these differences were larger at low Ce/Sw values than at high values. For example, the Ko values of TCB changed from 0.93 (HA) to 8.41 (LP) at Ce/Sw = 0.1, and from 1.0 (HA) to 29.5 (LP) at Ce/Sw = 0.001.

As characterized by elemental analysis, OPE, SEM and 13C-NMR, it was found that DP corresponded to Kerogen Type 3 and mainly contained low maturation and high paraffinic carbon huminite. LP corresponded to Kerogen Type 2 or 3 and had more complex composition such as inertinite, huminite, vitrinite and exinite, with smaller particle size and higher maturation than DP. HA approached the lignite coal rank. The behaviors of HOCs sorption on Chelsea soil correlated not only with OC content of three types SOMs (DP, LP and HA), but also with their structure and properties. HA have much higher affinity for HOCs than HA. LP had the largest KF and Koc and the largest deviations from linearity, which means sorption capacity and the heterogeneity of sorption

sites was greatest in all five tested sorbents for the same sorbate. Although DP also had a large sorption capacity, the larger nvalues imply that the heterogeneity of sorption sites was much smaller than LP.

The sorption of HOCs in soil is primarily regulated by SOM. Humic acid seemed To be the soft carbon domain while insoluble condensed organic matter (humin) seemed to be the hard carbon domain. Aliphatic structures of SOM, as observed for LP, could also contribute to both isotherm nonlinearity and large sorption capacity. Isotherm nonlinearity  was negatively related to hydrophobicity and molecular size, while sorption capacity increases with increase of these parameters.

2.3LITERATURE REVIEW 2:

2.3.1 Soil Organic Matter:

Soil Organic Matter by USDA,NRCS-United states of Department of Agriculture, Natural resources conservation service.

Soil organic matter (SOM) is the organic component of soil, consisting of three primary parts including small (fresh) plant residues and small living soil organisms, decomposing (active) organic matter, and stable organic matter (humus). Soil organic matter serves as a reservoir of nutrients for crops, provides soil aggregation, increases nutrient exchange, retains moisture, reduces compaction, reduces surface crusting, and increases water infiltration into soil.

Components vary in proportion and have many intermediate stages . Plant residues on the soil surface such as leaves, manure, or crop residue are not considered SOM and are usually removed from soil samples by sieving through a 2 mm wire mesh before analysis. Soil organic matter content can be estimated in the field and tested in a lab to provide estimates for Nitrogen, Phosphorus and Sulfur mineralized available for crop production and adjust fertilizer recommendations.

Soil organic matter impacts the rate of surface applied herbicides along with soil pH necessary to effectively control weeds. Soil organic matter impacts the potential for herbicide carryover for future crops, and amount of lime necessary to raise pH.

In-field Estimate for Soil Organic Matter

1. Soil Sampling: Soil organic matter is highly variable. At least 10 small samples are gathered randomly from an area that represents the soil type and management history from the surface

0-8 inch depth and placed in the small plastic bucket and mixed. You may also estimate organic matter at each sample site and average organic matter readings for the area you are assessing. Repeat for each sampling area.

2. Use moist soil. If the sample is dry moisten it.

3. Match the soil with the color that it most closely matches (Figure 3) organic matter color chart (or other method of estimating organic matter content). Record associated organic matter content in Table 3 and complete calculations in interpretations section of this document (suggest averaging several samples).

Record SOM content; estimate organic matter, carbon (C), nitrogen (N), phosphorus (P), and sulfur (S) contained in SOM; and estimate N, P and S mineralized annually in Table 3. Use information in

Tables 3 and 4 and follow example calculation to complete Table 3, compare management systems, and answer discussion questions.

Estimating Organic Material Needed to Increase Soil Organic Matter The term steady state is where the rate of organic matter addition from crop residues, roots and manure or other organic materials equals the rate of decomposition. If the rate of organic matter addition is less than the rate of decomposition, SOM will decline and, conversely if the rate of organic matter addition is greater than the rate of decomposition, SOM will increase.

2.4 LITERATURE REVIEW 3:

2.4.1 EFFECT OF ORGANIC AND MINERAL SOIL FRACTIONS ON SORPTION BEHAVIOUR:

Effect of organic and mineral soil fractions on sorption behaviour of chlorophenol and triazine micropollutants by Sanja STIPIČEVIĆ, Sanja FINGLER, and Vlasta DREVENKAR at Institute for Medical Research and Occupational Health, Zagreb, Croatia on Received in June 2008 Accepted in November 2008

This article compares the sorption behavior of 2,4,6-trichlorophenol, 2,3,4,6-tetrachlorophenol, pentachlorophenol, chlorotriazine atrazine, methylthiotriazine ametryn, methoxytriazine atratone, hydroxyatrazine, and didelakylated atrazine in a topsoil and an aquifer sediment before and after removal of sorbent organic matter and in humic acid.

Freundlich isotherm coefﬁcients Kf and 1/n and free energy change (∆G°) were calculated for all compounds in all sorbents. According to sorbent pH values, chlorophenolate anions and uncharged triazine species dominated in all sorption experiments with topsoil and aquifer sediment. In experiments with humic acid, chlorophenols, atrazine, and didealkylated atrazine existed almost completely as neutral species, whereas protonated species dominated for hydroxyatrazine, atratone, and ametryn.

In addition to a hydrophobic partition, sorption of all compounds in native soil and sediment sorbents includes speciﬁ c, more polar interactions, which greatly depend on sorbate acidity/ basicity, speciﬁ c properties of the sorbent organic matter and of mineral surface, as well as on the system pH. A signiﬁ cantly greater sorption intensity of all compounds in “organic-free” than in the native aquifer sediment conﬁ rmed the importance and possible dominance of mineral surface in the sorption process. Sorption intensity of chlorophenol and triazine compounds in humic acid was closely related to compound hydrophobicity. Greater sorption of almost completely protonated hydroxyatrazine than of the more hydropohobic but uncharged atrazine indicated different humic acid reaction sites for two compounds and consequently different sorption mechanisms.

Characteristics of the topsoil and aquifer sediment samples before (native sorbents) and after removal of organic phase (“organic-free” sorbents) by H2O2 treatment . Mineralogical analysis showed that quartz and carbonate minerals calcite and dolomite were dominant mineral species in the topsoil. The aquifer sediment was dominated by clay minerals, and the most abundant was montmorillonite. Native sorbents were similar in organic matter content but signiﬁcantly different in speciﬁc surface area (SSA); the SSA of the aquifer sediment was for one order of magnitude greater than of the topsoil SSA. The SSA of “organic-free” sorbents doubled compared to the native sorbents. Removal of organic phase also affected sorbent pH values. The native sorbents were neutral, but the “organic-free” topsoil became strongly alkaline.

2.5- LITERATURE REVIEW 4:

2.5.1 Correlation between Soil Organic Matter, Total Organic Matter and Water Content with Climate and Depths of Soil:

Correlation between Soil Organic Matter, Total Organic Matter and Water Content with Climate and Depths of Soil at Different Land use in Kelantan, Malaysia

A total of four sites distributed in different soils of Kelantan State, Malaysia  was identified for the study. Soils were collected by depth interval of 0-10cm, 1020cm and 20-30cm. The correlation of soil organic matter (SOM) content, total organic carbon (TOC) content, water content and soils texture for industrial area at Pengkalan Chepa, township of Kota Bharu district, agricultural area at Banggu and forested area in UMK, Jeli were investigated. These data sets were also correlated to temporal event in Kelantan State. Correlation analysis indicated that, generally, SOM and TOC concentration and soil classes had a positive correlation with temporal patterns and no significance effects with depth of the soils. The relationships between SOM content, TOC content and clay + silt content, were also studied.

The results showed that, SOM concentration was lower (P < 0.05) at Pengkalan Chepa area (1.96%) compared to Kota Bharu (2.06%), Banggu (2.77%) and Jeli (7.39%). At the same time, the TOC level also showed that Banggu area recorded the lowest concentration (0.42%) followed by Kota Bharu (0.71%), Pengkalan Chepa (0.76%) and Jeli (3.73%). The temporal factor (p < 0.05) showed that TOC content higher during dry season (1.76%) and lower during pre monsoon (0.48%) and lowest in monsoon season (0.25%). Similar results were obtained for SOM content, higher during dry season (4.00%) followed by pre monsoon (2.12%) and lowest in monsoon season (1.67%). The lowest TOC and SOM content in soil during monsoon season was believed to be due heavy rain which detaches all the organic matter from soil particles into river.

Sand was a dominant soil class in Pengkalan Chepa Industrial Park and southwest (SW) of Kota Bharu Township. The percentage of sand was higher towards 30cm depth for Pengkalan Chepa area from 94.44% to 95.41% but in contrast the percentage is lower towards 30cm for Kota Bharu township from 96.02% to 90.26%. The trend of soil texture (silt and clay) in Pengkalan Chepa shown that lesser percentage toward 30cm where silt from 3.26% to 2.52% and clay from 2.17% to 1.19%.

The different between soil texture percentages in Pengkalan Chepa almost constant at 1% (silt 0.74%, clay 0.98% and sand 0.95%).   SW of Kota Bharu township and Banggu area have almost the same trend where the silt and clay showed the increasing percentage from 0cm to 30cm depth. In contrast the sand at both area showed the decreasing trend towards 30cm depth. Banggu has a sandy loam of soil class and the township area has the same pattern to sandy loam class.

The remote area in Jeli District has constantly shown the sandy clay loam texture regardless the depth of soil. The Jeli area is typically very rich of particulate organic materials comprising fresh plants, as the area still covered of virgin’s forest.

Soils permeability for the studied areas are also classified based on hydrologic soil groups (HSGs) to indicate minimum rate of infiltration obtained for bare soil after prolonged wetting. For the area such as Pengkalan Chepa, Kota Bharu and Banggu showed HSG A with high infiltration rates greater than 0.30in/hr and have very low runoff potential of water and with moderately fine to fin texture. These soils have a low rate of water transmission (0.05-0.15in/hr).

2.6. Scope:

- Quantify the amount of organic carbon in soil

- Identify physical and chemical parameters of soil

- Leaching characteristics of soil by conducting lab scale column experiments

- Study on adsorption properties

2.7. Objective:

- To understand the influence of organic carbon in the soil from agricultural fields towards estimating the physical and chemical characteristics of soil

3. MATERIALS AND METHODS

3.1 General

Soil samples have been tested for soil organic carbon and Total Organic carbon content in soil by conducting various experiments such as physical and chemical characteristics of soil and column leaching experiment.

Study Area

The areas which were chosen for studying the carbon content in soil are the agricultural lands at two different places and  near the Sewage Treatment Plant (STP) of Bannari Amman Institute of Technology.

Sample collection

We have collected samples in 3layers at different depth of each 1feet in all the three areas. The samples were filled in air tight bag to avoid change of moisture content. Then the soil samples were marked as Sample A, B and C.

Sample Preparation

For analyzing TOC, initially 10g of each samples were mixed thoroughly with 0.1N of HCL, centrifuged and packed. 10g of each samples were mixed thoroughly with 0.1N of NaoH , centrifuged and packed. Again fully saturated samples weighing 10gcentrifuged and packed. The packed samples were sent to the Indian Institute of Technology, Madras for analyzing Total Organic Carbon. And for analyzing the concentration of chloride in soil by column leaching experiment, we created the set up of column tube and packed the sample A and conducted the experiment by discharging 100ppm of chloride solution at certain velocity.

3.2 Chemical tests for soil sample:

3.2.1 pH

Measurement of pH and electrical conductivity (EC) parameters provides valuable information for assessing soil condition for plant growth, nutrient cycling and biological activity. Soil and crop management practices have a significant effect on pH and EC and are considered good indicators of change. Elevated pH values can indicate potential losses of nitrate and subsequent water contamination. The tendency for soil acidification can suggest insufficient use of ammoniacal fertilizers and increased leaching losses

Measuring pH in salt water is more difficult than measuring pH in fresh water. The salt in seawater can interfere with the measurement. The pH paper tests are not suitable for measuring pH in salt water.

There are two methods commonly used to measure pH in an estuary. Colorimetric means to measure color. In the colorimetric method, chemicals are added to the water sample and those chemicals react with the water to produce a color change. The color indicates the pH of the water. The color can be measured visually or electronically. The colorimetric method does not work when the water is already colored because it contains dissolved organic matter or large amounts of algae. Colorimetric test kits are inexpensive and can cover a wide range of pH values.

Figure 3.1.pH meter

3.2.2 Electrical Conductivity

Electrical Conductivity is a very quick, simple and inexpensive  method that farmers and home gardeners can use to check the health of their soils. Whereas pH is a good indicator of the balance of available nutrients in your soil, Electrical Conductivity can almost be viewed as the quantity of available nutrients in your soil.

The way that Electrical conductivity can be measured is using an EC meter. The probe or sensor consists of two metal electrodes and a constant voltage is applied across the electrodes resulting in an electrical current flowing through the sample. Since the current flowing through the water is proportional to the concentration of dissolved ions in the water, the electrical conductivity can be measured. The higher the dissolved salt/ion concentration, the more conductive the sample and hence the higher the conductivity reading.

To get a soil extract we recommend a similar method as we do for testing pH so that both EC and pH measurements can be taken at the same time.

- Gather a fresh soil sample in a plastic zip-loc bag. Try to get a profile from the top 6” of soil that the plants will grow in and take care not to contaminate the sample by touching with anything.

- Open the bag and let it air-dry for a few hours until it is mostly dried.

- Mix the soil in the bag to ensure a homogenous sample and then use a sieve with approximate 2mm spacing to remove any large soil clumps.

- Measure out ½ of a cup of the dried soil and put into a glass beaker.

- Measure out ½ of a cup of distilled water and put this into the glass beaker with the soil.

- Stir the mixture gently for 30 seconds. Do not mix to harshly as you may destroy the humus structure and the soil may give up elements that it otherwise would not do in nature.

- Let the soil-water suspension stand for 30 minutes.

- Stir water gently again before taking the EC measurement.

- Insert the EC meter into the beaker and swirl it gently around in the soil-water extract.

- After approximately 30-60 seconds or after the EC reading has stabilized, read the digital display on your meter.

# 3.3 Physical tests for soil Sample:

# 3.3.1 Moisture Content:

3.3.1.1 Purpose:

This test is performed to determine the water (moisture) content of soils. Thewater content is the ratio, expressed as a percentage, of the mass of “pore” or“free” water in a given mass of soil to the mass of the dry soil solids.

3.3.1.2 Standard Reference:

ASTM D 2216 - Standard Test Method for Laboratory Determination of

Water (Moisture) Content of Soil, Rock, and Soil-Aggregate Mixture.

# Fig 3.2.Muffle Furnace

3.2.1.3 Test Procedure:

- Record the moisture can and lid number. Determine and record the

- mass of an empty, clean, and dry moisture can with its lid (MC)

- Place the moist soil in the moisture can and secure the lid.

- Determine and record the mass of the moisture can (now containing

the moist soil) with the lid (MCMS).

- Remove the lid and place the moisture can (containing the moist soil) in the drying oven that is set at 105 °C. Leave it in the oven overnight.

- Remove the moisture can. Carefully but securely, replace the lid on the moisture can using gloves, and allow it to cool to room temperature. Determine and record the mass of the moisture can and lid (containing the dry soil) (MCDS).

3.3.2 Bulk Density:

Bulk density is the weight of soil in a given volume. Soils with a bulk density higher than 1.6 g/cm3 tend to restrict root growth.Bulk density increases with compaction and tends to increase with depth.Sandy soils are more prone to high bulk density.Bulk density can be used to calculate soil properties per unit area (e.g. kg/ha).

The soil bulk density (BD), also known as dry bulk density, is the weight of dry soil (Msolids) divided by the total soil volume (Vsoil). The total soil volume is the combined volume of solids and pores which may contain air (Vair) or water (Vwater), or both (figure 1). The average values of air, water and solid in soil are easily measured and are a useful indication of a soils physical condition.
Soil BD and porosity (the number of pore spaces) reflects the size, shape and arrangement of particles and voids (soil structure). Both BD and porosity (Vpores) give a good indication of the suitability for root growth and soil permeability and are vitally important for the soil-plant-atmosphere system (Cresswell and Hamilton, 2002; McKenzie et al., 2004). It is generally desirable to have soil with a low BD (<1.5 g/cm3) (Hunt and Gilkes, 1992) for optimum movement of air and water through the soil.

Figure 3.3 Bulk Density

## 3.3.2.1 Measuring bulk density

Bulk density measurements can be done if you suspect your soil is compacted or as part of fertilizer or irrigation management plans see measurements at the same location over time and at different depths in the soil, for example at 10, 30 and 50 cm depths to look at both the surface soil and subsoil. It is also useful to measure the bulk density of when comparing management practices (e.g. cultivated vs. non-cultivated) as physical soil properties are often altered (Hunt and Gilkes, 1992).
The most common method of measuring soil BD is by collecting a known volume of soil using a metal ring pressed into the soil (intact core), and determining the weight after drying (McKenzie et al., 2004).

3.3.2.2 Sampling the soil

This method works best for moist soils without gravel. If sampling during summer, it is possible to wet the soil manually to keep bulk density core intact. To do this, place bottomless drum on the soil and fill with water, allowing to wet naturally for 24 hours.

Using the appropriate tools (see information box), prepare an undisturbed flat horizontal surface in the soil with a spade at the depth you wish to sample. Push or gently hammer the steel ring into the soil. A block of wood may be used to protect the ring. Avoid pushing the ring in too far or the soil will compact. Excavate around the ring without disturbing or loosening the soil it contains and carefully remove it with the soil intact (figure 2). Remove any excess soil from the outside the ring and cut any plants or roots off at the soil surface with scissors). Pour the soil into the plastic bag and seal the bag, marking the date and location where the sample was taken. Common sources of error when measuring BD are disrupting the soil while sampling, inaccurate trimming and inaccurate measuring of the volume of the ring. Gravel can make trimming the core difficult and give inaccurate values, and it is best to take more samples to decrease error in this way.

The critical value of bulk density for restricting root growth varies with soil type (Hunt and Gilkes, 1992) but in general bulk densities greater than 1.6 g/cm3 tend to restrict root growth (McKenzie et al., 2004). Sandy soils usually have higher bulk densities (1.3–1.7 g/cm3) than fine silts and clays (1.1 – 1.6 g/cm3) because they have larger, but fewer, pore spaces. In clay soils with good soil structure, there is a greater amount of pore space because the particles are very small, and many small pore spaces fit between them. Soils rich in organic matter (e.g. peaty soils) can have densities of less than 0.5 g/cm3.
Bulk density increases with compaction

3.3.3 Particle size analysis

3.3.3.1Test  Procedure:

- Take a representative oven dried sample of soil that weighs about 500 g. ( this is normally used for soil samples the greatest particle size of which is 4.75 mm)

- If soil particles are lumped or conglomerated crush the lumped and not the particles using the pestle and mortar.

- Determine  the mass of sample accurately Wt (g)

- Prepare a stack of sieves. sieves having larger opening sizes (i.e. lower numbers) are placed above the ones having smaller opening sizes (i.e. higher numbers). The very last sieve is #200 and a pan is placed under it to collect the portion of soil passing #200 sieve.  Here is a full set of sieves. (#s 4 and 200 should always be included)

- Draw graph of log sieve size vs. % finer. The graph is known as grading curve. Corresponding to 10%, 30% and 60% finer, obtain diameters from graph these are D10,D30, D60, using these obtain Cc and Cu which further represent how well the soil is graded i.e. whether the soil is well-graded, gap-graded or poorly graded.

Figure 3.4 Sieve for particle size analysis

## 3.3.4 PERMEABILITY TEST

3.3.4.1 Falling Head Method

- Prepare the soil specimen as specified.

- saturate it. Deaired water is preferred.

- Assemble the permeameter in the bottom tank and fill the tank with water.

- Inlet nozzle of the mould is connected to the stand pipe. Allow some water to flow until steady flow is obtained.

- Note down the time interval t for a fall of head in the stand pipe h

- Repeat step 5 three times to determine for the same head.

- Find by collecting q for the stand pipe. weigh it correct to 1 gm and find a from q/h=a.

- Therefore the coefficient of permeability

- During test there should be no volume change in the soil, there should be no compressible air present in the voids of soil i.e. soil should be completely saturated. The flow should be laminar and in a steady state condition. Coefficient of permeability is used to assess drainage characteristics of soil, to predict rate of settlement founded on soil bed.

3.3.5 Porosity:

Porosity or pore space is the amount of air space or void space between soil particles. Infiltration, groundwater movement, and storage occur in these void spaces. The porosity of soil or geologic materials is the ratio of the volume of pore space in a unit of material to the total volume of material.

Loose, porous soils have lower bulk densities and greater porosities than tightly packed soils. Porosity varies depending on particle size and aggregation. It is greater in clayey and organic soils than in sandy soils. A large number of small particles in a volume of soil produces a large number of soil pores. Fewer large particles can occupy the same volume of soil so there are fewer pores and less porosity.

Compaction decreases porosity as bulk density increases. If compaction increases bulk density from 1.3 to 1.5 g/cm3, porosity decreases from 50 percent to 43 percent. Aggregation also decreases porosity because more large pores are present as compared to single clay and silt particles that are associated with smaller pores.

Pores of all sizes and shapes combine to make up the total porosity of a soil. Porosity, however, does not tell us anything about the size of pores.

Porosity of surface soil typically decreases as particle size increases. This is due to soil aggregate formation in finer textured surface soils when subject to soil biological processes.

Aggregation involves particulate adhesion and higher resistance to compaction. Typical bulk density of sandy soil is between 1.5 and 1.7 g/cm3. This calculates to a porosity between 0.43 and 0.36. Typical bulk density of clay soil is between 1.1 and 1.3 g/cm3.

This calculates to a porosity between 0.58 and 0.51. This seems counterintuitive because clay soils are termed heavy, implying lower porosity. Heavy apparently refers to a gravitational moisture content effect in combination with terminology that harkens back to the relative force required to pull a tillage implement through the clayey soil at field moisture content as compared to sand.

3.3.5.1 MEASUREMENT METHODOLOGY

Determination of the effective porosity, pe of soils can be accomplished indirectly by measuring the total porosity, pt, and the field capacity,r, and then calculating pe from Equation 4.4. The total porosity is obtained indirectly by measuring the soil densities according to the method described in Section 3.2. To determine the field capacity of the soils, the soil sample is first saturated with water and is then allowed to drain completely under the action of gravity until it gets to its irreducible saturation. The value ofr can then be obtained according to the methods used for measuring volumetric water content

3.4 SAMPLE EXTRACTION FOR TOC ANALYSIS

Organic matter in soils and sediments is widely distributed over the earth surface occurring in almost all terrestrial and aquatic environments (Schnitzer, 1978). Soils and sediments contain a large variety of organic materials ranging from simple sugars and carbohydrates to the more complex proteins, fats, waxes, and organic acids. Important characteristics of the organic matter include their ability to: form water-soluble and water insoluble complexes with metal ions and hydrous oxides; interact with clay minerals and bind particles together; sorb and desorb both naturally-occurring and anthropogenically-introduced organic compounds; absorb and release plant nutrients; and hold water in the soil environment. As a result of these characteristics, the determination of total organic carbon (a measure of one of the chemical components of organic matter that is often used as an indicator of its presence in a soil or sediment) is an essential part of any site characterization since its presence or absence can markedly influence how chemicals will react in the soil or sediment. Soil and sediment total organic carbon (TOC) determinations are typically requested with contaminant analyses as part of an ecological risk assessment data package. TOC contents may be used qualitatively to assess the nature of the sampling location (e.g., was a depositional area) or may be used to normalize portions of the analytical chemistry data set (e.g., equilibrium partitioning).

The test procedure was carried by the Indian Institute of Technology, madras which is briefly discussed in result and discussion.

3.5 COLUMN EXPERIMENT

Column experiment is done to test the absorption capacity of soil which is done by Carbon present in it.

3.5.1 Procedure:

- Sample A was tightly packed in a PVC pipe and both the ends were sealed using  pipe caps.

- Distiled water is mixed with some amount of chloride and filled in a burette.

- It is then made to pass through the pipe which is filled with soil sample.

- It passed through the soil sample and gets collected in a measuring jar in the opposite end .

- The water in the measuring jar was tested for chloride by titrating against silver  nitrate by using the indicator Potassium chromate.

- And the amount of chloride was estimated.

- The graph was indicating the break point.

The more the chlorine content the less the adsorption capacity of soil and hence carbon present in soil is also too low.

4. RESULTS AND DISCUSSION

4.1 GENERAL

Based on the detailed analysis of soil samples for its various physical-chemical characteristics, the adsorption properties of the soil organic matter can be evaluated. This chapter gives the observed values of various experiments and their corresponding results obtained in the laboratory. Physical Characteristics like Moisture Content, Bulk Density, Particle Size Distribution, Porosity and Permeability and Chemical  Characteristics like pH and Electrical Conductivity were being  analysed in the corresponding three samples.

4.2.CHEMICAL TEST OF SOIL

4.2.1.pH

The soil pH is a measure of the acidity or alkalinity in soils. pH is defined as the negative logarithm (base 10) of the activity of hydronium ions (H+ or, more precisely, H3O+aq) in a solution. Acidity in soils comes from H+ and Al3+ ions in the soil solution and sorbed to soil surfaces. Alkaline soils have a high saturation of base cations (K+, Ca2+, Mg2+ and Na+). This is due to an accumulation of soluble salts which are classified as either saline soil, sodic soil, saline-sodic soil or alkaline soil.The observed values for pH are shown in the Table 4.1.

Table 4.1 : Observed values for  pH

| S.No | Samples | pH | Nature |
| --- | --- | --- | --- |
| 1 | A | 6.37 | Neutral |
| 2 | B | 6.61 | Neutral |
| 3 | C | 6.95 | Neutral |

. From the table 4.1, all the three soil samples are considered to be Neutral. Though the pH values of all the three samples are observed to be less than 7, the presence of H+ ion is so negligible that the samples are considered as Neutral..

4.2.2.ELECTRICAL CONDUCTIVITY

Electrical conductivity (EC) is the most common measure of soil salinity and is indicative of the ability of an aqueous solution to carry an electric current. Plants are detrimentally affected, both physically and chemically, by excess salts in some soils and by high levels of exchangeable sodium in others. Soils with an accumulation of exchangeable sodium are often characterized by poor tilth and low permeability making them unfavorable for plant growth.The  Observed values for Electrical Conductivity are shown in Table 4.2.

Table 4.2: Observed values for Electrical Conductivity

| S.No | Samples | Electrical Conductivity |
| --- | --- | --- |
| 1 | A | 0.328 |
| 2 | B | 0.436 |
| 3 | C | 0.409 |

There are so many variables affecting the EC level that it almost depends on your individual conditions which if you analyze over time, will give you a meaningful set of data based on the performance of your crops and the changes you have made to your fertility program.

As a general guideline however, a good soil EC level will be somewhere above 200 µS/cm and 1200 µS/cm (1.2 MS/cm). Any soils below 200 means there is not enough nutrients available to the plant and could perhaps show a sterile soil with little microbial activity.

4.3. PHYSICAL TESTS

4.3.1. MOISTURE CONTENT

Moisture content is the quantity of water contained in soil. Water content is used in a wide range of scientific and technical areas, and is expressed as a ratio, which can range from 0 (completely dry) to the value of the materials  porosity at saturation.

Table 4.3. Observed values of Moisture Content

| S.No | Samples | Weight of the Soil + Moisture Content (g) | Weight of the Soil after Evaporation (g) | Final Moisture Content (%) |
| --- | --- | --- | --- | --- |
| 1 | A | 10 | 9.1801 | 8.2 |
| 2 | B | 10 | 9.2736 | 7.26 |
| 3 | C | 10 | 8.9567 | 10.43 |

From the Table 4.3,  it is found that the soil samples have an optimum moisture content as the samples are near the agricultural field.

4.3.2. BULK DENSITY

Bulk Density is the mass of many particles of the material divided by the total volume they occupy. The total volume includes particle volume, inter-particle void volume, and internal pore volume.The Observed values of Bulk Density are shown in the Table 4.4.

Table 4.4 : Observed values of Bulk Density

| S.No | Samples | Weight of (Soil + Measuring Jar) (Kg) | Weight of Soil (Kg) | Bulk Density(g/cc) |
| --- | --- | --- | --- | --- |
| 1 | A | 0.481 | 0.375 | 1.5 |
| 2 | B | 0.427 | 0.321 | 1.284 |
| 3 | C | 0.429 | 0.323 | 1.292 |

The Nominal values of the Bulk Density are shown in the below table,

| Bulk density(g/cc) | Pore space(%) |
| --- | --- |
| 1.6 | 40 |
| 1.4 | 47 |
| 1.3 | 50 |
| 1.1 | 58 |

Thus the corresponding values of Bulk Desity are shown in the Table 4.4.  The Pore Space of the samples are found in the range of about 50 % -  40 %.

4.3.3. PARTCLE SIZE ANALYSIS

The Partcle Size Analysis of a material can be important in understanding its physical and chemical properties. It affects the strength and load-bearing properties of rocks and soils. It affects the reactivity of solids participating in chemical reactions, and needs to be tightly controlled in many industrial products such as the manufacture of printer toner, cosmetics, and pharmaceutical products. The corresponding D60, D30, D10 of the Samples A, B,and C are shown in the Table 4.5.

Sample A

| S.No | Sieve Opening (mm) | Weight Retained (gm) | Percentage Retained | Cumulative Percentge | Percentage finer |
| --- | --- | --- | --- | --- | --- |
| 1 | 4.75 | 48 | 10.14 | 10.14 | 89.86 |
| 2 | 2.36 | 59 | 18.81 | 28.95 | 71.05 |
| 3 | 1.98 | 85 | 17.97 | 46.92 | 53.08 |
| 4 | 0.6 | 52 | 10.99 | 57.91 | 42.09 |
| 5 | 0.3 | 102 | 21.56 | 79.47 | 20.53 |
| 6 | 0.15 | 75 | 15.85 | 95.32 | 4.68 |
| 7 | 0.075 | 12 | 2.53 | 97.85 | 2.15 |
| 8 | Pan | 10 | 2.11 | 99.96 | 0.04 |

Sample B

| S.No | Sieve Opening (mm) | Weight Retained (gm) | Percentage Retained | Cumulative Percentge | Percentage finer |
| --- | --- | --- | --- | --- | --- |
| 1 | 4.75 | 85 | 17.06 | 17.06 | 82.94 |
| 2 | 2.36 | 123 | 24.69 | 41.75 | 58.25 |
| 3 | 1.98 | 86 | 17.26 | 59.01 | 40.99 |
| 4 | 0.6 | 63 | 12.65 | 71.66 | 28.34 |
| 5 | 0.3 | 76 | 15.26 | 86.92 | 13.08 |
| 6 | 0.15 | 50 | 10.04 | 96.96 | 3.04 |
| 7 | 0.075 | 7 | 1.41 | 98.37 | 1.63 |
| 8 | Pan | 8 | 1.61 | 99.98 | 0.02 |

Sample C

| S.No | Sieve Opening (mm) | Weight Retained (gm) | Percentage Retained | Cumulative Percentge | Percentage finer |
| --- | --- | --- | --- | --- | --- |
| 1 | 4.75 | 92 | 18.4 | 18.4 | 81.6 |
| 2 | 2.36 | 139 | 27.8 | 46.2 | 53.8 |
| 3 | 1.98 | 112 | 22.4 | 68.6 | 31.4 |
| 4 | 0.6 | 78 | 15.6 | 84.2 | 15.8 |
| 5 | 0.3 | 58 | 11.6 | 95.8 | 4.2 |
| 6 | 0.15 | 16 | 3.2 | 99 | 1 |
| 7 | 0.075 | 2 | 0.4 | 99.4 | 0.6 |
| 8 | Pan | 3 | 0.6 | 100 | 0 |

Table 4.5. Corresponding values of  D60, D30, D10 of the Samples

|  | A | B | C |
| --- | --- | --- | --- |
| D60 (mm) | 1.2 | 0.35 | 0.12 |
| D30 (mm) | 2.5 | 0.66 | 0.25 |
| D10 (mm) | 2.8 | 1.1 | 0.48 |

From the values of Coefficient of Curvature and Uniformity Coefficient, it can be said that the samples are Well Graded Clay Soil.

4.3.4. PERMEABILITY OF SOIL – FALLING HEAD METHOD

Permeability is a measure of the ability of a to allow fluids to pass through it. The permeability of a medium is related to the porosity, but also to the shapes of the pores in the medium and their level of connectedness.The Table 4.6.

Table 4.6 : Observed values for Permeability – Falling head

| S.No | Samples | Initial Head (H1) (cm) | Final Head (H2) (cm) | Time taken for water level to fall from H1 to H2 | Coefficient of Permeability (KT) (m/s) |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | A | 151.5 | 141.5 | 2 | 0.01365 |  |
| 2 | B | 141.5 | 131.5 | 3 | 0.00977 |  |
| 3 | C | 131.5 | 121.5 | 3 | 0.01054 |  |

Thus, from the Nominal values and the test samples, it had been observed that all the three samples are considered to be highly permeable which indicates that it allows water easily.

4.3.5. POROSITY

Porosity is a measure of the void (i.e. "empty") spaces in a material, and is a fraction of the volume of voids over the total volume, between 0 and 1, or as a percentage between 0 and 100%. The Observed values for Porosity are shown in the Table 4.7.

Table 4.7 : Observed values for Porosity

| S.No | Samples | Initial Volume of Water (ml) | Final Volume of Water (ml) | Volume of Water Consumed (ml) | Porosity (%) |
| --- | --- | --- | --- | --- | --- |
| 1 | A | 10 | 7 | 3 | 5 |
| 2 | B | 10 | 8 | 2 | 2.7 |
| 3 | C | 10 | 6 | 4 | 19.4 |

It is found that the values of Porosity for all the three samples are less than 10%.It can be further observed that Samples A and  C are highly porous and the Sample B has low porosity.

4.4 Estimation of TOC

Total Organic Carbon(TOC) is used to reveal the interaction between the dissolved organic carbon released from manure with pesticides may increase or decrease the pesticide movement through soil into groundwater .TOC glues aggregates together and stabilises the soil structure making soil resistant to erosion, but porous enough to allow air, water and other plant roots to move through the soil. Table 4.8 explains the result values of Total Organic Carbon(TOC) present in the samples.

Table 4.8 Results for Total Organic Carbon

| Sample | Extractable Dissolved TOC (mg/L) | Percentage of TOC in soil |
| --- | --- | --- |
| A | 32.1 | 2.6 |
| B | 65.12 | 2.3 |

From the Table 4.8, based on the TOC values, it is found that the Sample A is Clay Loam and Sample B is Sandy Loam.

4.5. Column Leaching Experiment

Column Leaching Experiment is done to test the adsorption capacity of the soil which is done by Carbon present in the soil.Table 4.9 shows the burette reading and Table 4.9 shows Concentration of Chloride for the respective time interval.

Table 4.9  Burette Reading for the Column Experiment

| S.no | Time taken for 10 ml (min) | Burette Reading (ml) |  |
| --- | --- | --- | --- |
|  |  | Initial | Final |
| 1 | 47 | 0 | 4.7 |
| 2 | 11 | 4.7 | 7.7 |
| 3 | 19.7 | 7.7 | 10.6 |
| 4 | 3.55 | 10.6 | 13.8 |
| 5 | 7.29 | 13.8 | 16.2 |
| 6 | 8.35 | 16.2 | 18.4 |
| 7 | 6.21 | 18.4 | 20.9 |
| 8 | 7.29 | 20.9 | 22.8 |
| 9 | 12.33 | 22.8 | 24.7 |

Table 4.10: Concentration of Chloride for the respective time interval

| Time taken for 10 ml (min) | Concentration | C/C0 |
| --- | --- | --- |
| 47 | 233.261 | 1 |
| 77.7 | 143.927 | 0.329631325 |
| 88.54 | 119.2 | 0.144078162 |
| 96.89 | 109.18 | 0.068887371 |
| 110.39 | 94.297 | 0 |
| 122.72 | 94.297 | 0 |

From the Graph, it is found that

1. Chloride concentration in the soil is found to be about 230 mg/L.

By conducting the leaching experiment, the movement of dissolved ions through the soil can be determined. It has been observed that the background chloride concentration in the soil is significant compared to the proposed tracer analysis. By conducting the continuous leaching experiment, the maximum prevailing chloride concentration has been observed to be 233.261 mg/L which has came out after  47 minutes.

As the time progresses, the concentration was found to be decreased from 143.927 mg/L to 100 mg/L within 76.12 minutes.

At the end of 122.72 minutes, the effluent chloride concentration has been reduced to  about 100 mg/L. This shows that the background concentration has been exhausted within 122.72 minutes and the added chloride has reached the end of the column.

5. CONCLUSION

The Soil properties were studied on both physical and chemical characteristics and we have concluded that it is Sandy Loam soil and the other sample was Clayey loam.

TOC (Total Organic Carbon) was done to estimate the amount of carbon present in soil samples. As, carbon plays an important role in the adsorption mechanism of soil.

Column experiment was done to test the absorption capacity of soil which was done by carbon present in it. The more the chloride content obtained after the column experiment by estimating the chloride, the less the adsorption capacity of soil and hence ,we concluded that carbon present in soil will also be too low and vice versa.

TOC is high for Clayey soil indicating it has more SOM and good adsorbent capacity. It is quite low for Sandy Loam soil indicating it has low SOM comparing Clayey soil and its adsorbent capacity will also be low comparing the clayey soil sample.

Hence, we conclude that if Soil Organic Matter (SOM) is high then the Total Organic Carbon (TOC) will also be high indicating it has a high adsorption capacity.

6. REFERENCES

1. Azlan , Aweng ,Ibrahim ,Noorhaidah(2012) 'Correlation between TOC,SOM & water content with climate & depths of soil at different land use in Kelantan,Malaysia',J.Appl.Sci.Environ.Manage-Vol 16(4)353-358.

2. Frank K.Cameron(2005) 'A comparison of the organic matter in different soil types', Journal of the American Chemical Society- Vol  27- issue 3.

3. Huang K, Tonde N & M.Th.Van Genuhten(1994)'Experimental Investigation of solute transport in large ,homogeneous & heterogeneous ,saturated soil columns'.

4. Joseph R McAuliffe (2009) 'Landscape evolution, soil formation and ecological patterns and processes in Sonoran desert Bajadas', Ecological Monographs,

Vol 64-no 2.

5. Nelson,D.W;Sommers L.E(1996) 'Total carbon, organic carbon and organic matter-methods of soil analysis'-Part 3.

6. Pierre-Yves Hicher(1996) 'Elastic properties of soils'- J. Geotech. Eng., 10.1061/(ASCE)0733-9410(1996)122:8(641), 641-648.

7. United States Department of Agriculture-Natural Resources Conservation Service(2015)' Soil Organic Matter'.

8. United States Department of Agriculture-Natural Resources Conservation Service(2015)'Soil Electrical Conductivity'.

9.European Union 7th framework programme(2007)'Guide lining protocol for soil column experiments assessing fate & transport of trace organics', for the research project DEMEAU(FP7/2007-13) .

10. Earth Science(http://study.com/academy/lesson)

11. Total Organic Carbon(http://www.soilquality.org.au)

12. Soil quality considerations in the selection of sites for aquaculture(http://www.fao.org/docrep/field/003/ac172e/ac172e04.htm)

13. ‘Range of permeability for various soils’-lec-11 slide 009(http://courses.washington.edu)

14. ‘Miscellaneous soil properties’ (http://ftp.fao.org)

7.APPENDIX

7.1Moisture Content:

Moisture Content = (Initial weight - Final Weight)*100/Initial Weight

for Sample A,       = (10-9.1801) *100 /10

=  8.2%

for Sample B,	= 7.26%

for Sample C,         = 10.43%

7.2 Bulk Density:

Weight of the Measuring Jar = 0.106 Kg

Volume of Cylinder = 250 ml

Bulk Density, ᵨ = (Weight / Volume)

Sample A = (0.375 * 100 / (250)) = 1.5 g/cc

7.3 Particle size Analysis:

D60 = Soil diameter at 60% finer =1.2 mm

D30 = Soil diameter at 30% finer = 0.35 mm

D10 = Soil diameter at 10% finer = 0.12 mm

Effective Size = D10 = 0.12 mm

Uniformity Coefficient = D60/D10

=1.2 / 0.12

= 10 mm

Coefficient of Curvature = (D30)2 / (D60 * D10)

= (0.35)2/(1.2 * 0.12)

= 0.851mm

Sample B

D60 = Soil diameter at 60% finer =2.5 mm

D30 = Soil diameter at 30% finer = 0.66 mm

D10 = Soil diameter at 10% finer = 0.25 mm

Effective Size = D10 = 0.25 mm

Uniformity Coefficient = D60/D10

=2.5 / 0.25

= 10 mm

Coefficient of Curvature = (D30)2 / (D60 * D10)

= (0.66)2/(2.5 * 0.25)

= 0.696 mm

Sample C

D60 = Soil diameter at 60% finer =2.8mm

D30 = Soil diameter at 30% finer = 1.1 mm

D10 = Soil diameter at 10% finer = 0.48 mm

Effective Size = D10 = 0.48 mm

Uniformity Coefficient      = D60/D10

=2.8 / 0.48

= 5.83 mm

Coefficient of Curvature    = (D30)2 / (D60 * D10)

= (1.1)2/(2.8* 0.48)

= 0.900 mm

7.4. Porosity

Porosity,n		= Volume of  Void vv / Total volume VT

= 1- (Bulk Density / Particle Density)

Particle Density  = Weight of Soil/(Initial Volume-Final Volume)

= 10 /(10-3) = 1.428 g/cc

for Sample A ,       =  1-(1.5-1.428)

n   = 0.05

for Sample B ,   n   = 0.027

for Sample C ,   n   = 0.194

7.5.Column Leaching Experiment

[(Volume of AgNo3 *Normality of  AgNo3   								*Equivalent weight of chloride)*1000]

Concentration of chloride = --------------------------------------------------------------

Volume of Sample

= (4.7 * 0.014 * 35.45 *1000)/10

Concentration C1  = 233.261mg/l

C2= 148.89 mg/l

C3   = 143.927 mg/l

C4   = 158.816 mg/l

C5   = 119.2 mg/l

C6   = 109.18 mg/l

C7  = 124.075 mg/l

C8 = 94.297 mg/l

C9   = 94.297 mg/l

7.6.Dispersivity Calculation:

u=Q/nA

Q=1 ml/s   =1cm3/s

Q=60cm3/min

n=0.027

A=(π/4)*d2

=50.28 cm2

u=60/50.28=1.19cm/min

x=28 cm

Break Through Curve Dispersivity coefficient Calculation:

t 0.16 =2 min

t 0.84 =32 min

D=1/8{[(x-ut 0.16 )/√t 0.16 ]- [(x-ut 0.84 )/√t 0.84 ]}

=2.48cm2/min

Leaching Curve Dispersivity Coefficient Calculation:

t 0.16 =84 min

t 0.84 =60 min

D=1/8{[(x-ut 0.16 )/√t 0.16 ]- [(x-ut 0.84 )/√t 0.84 ]}

=0.281 cm2/min